A Short Introduction to ATM Concepts

Wandel & Goltermann Technologies, Inc.

Electronic Measurement Technology

Research Triangle Park, NC 27709 USA

Phone: 919-941-5730   FAX: 919-941-5751

Jack Krupicka

Product Manager

E-Mail: krupicka@wg.com

4/94 Ver 1.0

Introduction to ATM

What is this ATM thing, and how do I get my money?

ATM for Telecommunications is Asynchronous Transfer Mode, (not Automatic Teller Machine). Division 2 will have ATM testing products available in calendar year 94, so this is the right time to start learning. ATM is a technology that has transport, switching, network management, and customer services built into it right from the start. The details of these, and how they can be tested, will be explored in this tutorial series.

ATM is a "Cell Relay" technology

In general, ATM means that traffic is carried in small, fixed-length packets called cells. International standards call for a 53-byte cell, 5 bytes of which is a header, and 48 bytes are payload. The fixed length cell gives some key advantages from earlier packet technology. The first is that the short cells can be switched in hardware, so that ATM can be switched quickly and economically. The second advantage is that queueing delays caused by long, variable-length frames can be reduced to the wait time for a 53-byte cell, allowing time-dependent voice and video to be transported.

Why do we need a new technology?

To answer this, let's first take a quick look at the two transport technologies available today: our traditional digital telephony system using Time Domain Multiplexing (TDM) technology, and packet switching systems like X.25 or Frame Relay. What are the advantages and disadvantages of each? The answers to these questions will point us directly to ATM technology.

Time Domain Multiplexing

TDM (also referred to as Synchronous Tranfer Mode, or STM) systems provide fixed bandwidth channels, illustrated here. For signals requiring Continuous Bit Rate (CBR) transport such as traditional voice and video this is ideal, but for computers with bursty traffic, this is very wasteful of network resources. Also, the granularity of the available bandwidth is very coarse. For instance, in North America, there is no standard transport bandwidth between 1.5Mbs and 45Mbs. An application requiring, say 10Mbs to accommodate bursts for Ethernet-to-Ethernet connectivity, needs a very expensive, dedicated 45Mbs transport system!

Packet Switching

Packet transfer technologies provide statistical multiplexing of frames onto the transport medium. This is ideal for bursty, Variable Bit Rate (VBR) data transport, but for constant bit rate voice and video it can cause a relatively high fixed delay as well as uncertain queueing delays.

The TDM system described above was designed for voice transport, and is only marginally effective for data transport. In the same way, packet switches were designed for data transport, and are even less capable of effective CBR transport.

ATM can provide both CBR and VBR transport

ATM Cell Relay is a packet switching technology that can provide high quality CBR service as well as economical VBR transport. Unlike TDM systems, it can provide an almost unlimited range of bit rates. In the example shown, CBR service is provided by allocating equally spaced slots. Data cells can fill up the remaining bandwidth.

ATM Cells

Looking at ATM from the Inside-Out

We discussed a few of the "big-picture" ATM issues in the previous article.  Now let's look from the inside-out, at the one thing that truly differentiates ATM: the cell. Each field of the cell header will lead us to an important understanding of ATM cell transport and switching.

VPIs/VCIs

First notice the two fields VPI & VCI which contain the Virtual Path and Virtual Channel Identifiers. Paths and Channels are Virtual because the ATM network provides transport and switching services that can mimic existing TDM Constant Bit Rate and Packet Switched Variable Bit Rate Channels. Paths and Channels are separated because ATM networks provide a two-tiered approach to switching: Channels carry customer data, while Paths carry groups of Channels. A customer can bundle links (Channels) between two fixed sites into a single Path, allowing the ATM network to ignore the multiplicity of Channels, and thus provide the service for less cost. The DS3 ATM Transport figure shows the relationship of the DS3 transport pipe, DS3 framing, PLCP framing, VPIs, and VCIs.

VPI/VCI Switching

ATM switching is performed by routing incoming VPI/VCIs to different outgoing VPI/VCIs. Typically, this is performed using a hardware-based lookup table. The incoming cells have their VPI/VCIs overwritten with the new values, and the cells are then routed to the desired output port. Notice then that VPI/VCIs have only local significance, since they change at each switch point. So how does a customer's data reach the desired destination? In early ATM networks, virtual paths and channels are permanent (Permanent Virtual Channels or PVCs). A PVC is created in a provisioning process that includes modifying the routing table of each ATM switch in the data path to provide the desired connection. Later ATM networks will use Signaling procedures to establish Switched Virtual Channels or SVCs, that can be set up or taken down when needed. Whether PVC or SVC, all ATM channels are connection oriented, by which we mean that a "connection" must be established in the ATM network before data can be sent.

Header Error Control

The ATM Cell Header is made extremely robust for transport by the Header Error Control (or HEC) field. This 8-bit field provides a checksum for the other 4 bytes of the header. The checksum can be used to detect errors with a high probability, or it can be used to correct single errors. For testing and monitoring purposes, the HEC is an easy method to determine line quality.

Generic Flow Control

The Generic Flow Control field occurs only in cells at the User-Network-Interface or UNI. Its detailed use is still in the process of standardization, but its basic purpose is to provide metering  and control of data entering the ATM Network before it enters, since the ATM Network does not provide store-and-forward buffering.

Cell Loss Priority

Cell Loss is the natural reaction of ATM switches to congestion. Unlike TDM systems that reserve time-slots for all services, ATM networks are very flexible. High-priority video or voice services can reserve specific cell positions, while LAN-to-LAN data services will reserve system capacity. Data is, however, bursty in nature, and maximum system efficiency can only be achieved by taking advantage of this. The Cell Loss Priority bit in each cell is settable by the ATM Network. An early use of this will be to tag those cells entering the network that exceed the customer's committed information rate (CIR). This will allow the network to discard those cells if needed.

Payload Type Identifier

The main purpose of the PT Identifier is to discriminate between user cells and non-user cells such as Operation and Maintenance (or OAM) cells. OAM cells provide Layer Management features for the ATM Layer, and will be discussed in a future article.

| PTI Coding | Interpretation |
| --- | --- |
| 000 | User data cell, congestion not experienced, SDU-type = 0 |
| 001 | User data cell, congestion not experienced, SDU-type = 1 |
| 010 | User data cell, congestion experienced, SDU-type = 0 |
| 011 | User data cell, congestion experienced, SDU-type = 1 |
| 100 | Segment OAM F5 flow related cell |
| 101 | End-to-end OAM F5 flow related cell |
| 110 | Reserved for future traffic control and resource management |
| 111 | Reserved for future functions |

The ATM Forum UNI Specification

What is the ATM Forum, and where are the real ANSI & CCITT Specs?

A short history lesson may help.  In 1988 CCITT (now ITU) decided to base future B-ISDN (Broadband ISDN) service on ATM technology. As described in the previous article, ATM can provide transport and switching services equally well for both data and voice/video. B-ISDN/ATM was envisioned as a service/technology that would be researched during the 90s, and implemented well past the turn of the century. The ANSI working group T1S1.5 worked on this as well. However,  ATM technology has also been adopted by private network vendors who view it as the first technology where LAN and WAN can come together to provide data, voice, and video services. Four companies --  Adaptive, Northern Telecom, Sprint, and Cisco Systems founded the ATM Forum to ensure interoperability between the public and private ATM implementations. Their purpose was not to create standards, but to agree on which sets of specifications and options within them to use. Since August 1991 those four members have grown to nearly 400. Though not an official standards body, this group is making defacto ATM standards for the industry.

Physical Layer Interfaces

An advantage of ATM is that ATM cells can be transported over almost any medium. The ATM Forum has defined the first four interfaces for the public and private User-Network Interface (UNI). They are 155 Mbps SONET, 45Mbps DS3, 100Mbps Multimode Fiber, and a second 155Mbps interface based on Fiber Channel. The SONET and DS3 interfaces are long-haul transport systems suitable for the User-to-Public-Network connection, while the other two are short/medium haul suited for LAN or campus backbones. Each physical layer standard is subdivided into a Physical Media Dependent Sublayer (PMD) and a Transmission Convergence Sublayer (TC).  The PMD for each of the four interfaces comes directly from existing standards. The TC combines existing protocols required for network transport, such as DS3 and SONET framing, with new functions that transport cells within the payload of the existing system.

DS3 Physical Layer

Standard asynchronous DS3 is made suitable for transporting cells by adding another layer of framing to the normal DS3 Frame pattern. This additional layer is called the Physical Layer Convergence Protocol (PLCP). Loosely modeled on SONET framing, PLCP provides a 125 usec multiframe, Cell Delineation, and many state of the art framing features like Bit Interleaved Parity, and Path Status that are not available in the DS3 framing structure.

Interim Local Management Interface (ILMI)

Easy management of ATM networks is a crucial feature. Central administrative authorities must be able to monitor the behavior and status of the network. The following is taken from the ATM Forum UNI Specification :

ITU and ANSI are currently working on TMN standards, and are in large part considered "for further study" with respect to ATM. In the interim period, until these standards are available, the Simple Network Management Protocol (SNMP) and an ATM UNI Management Information Base (MIB) will be required to provide any ATM user device with status and configuration information concerning the Virtual Path and Channel Connections available at its UNI.

The types of management information that will be available in the ATM UNI MIB are: Physical Layer, ATM Layer, ATM Layer Statistics, Virtual Path (VP) Connections, Virtual Channel (VC) Connections, and Address Registration Information.

UNI Signaling

ATM is a connection-oriented technology, requiring routing to be established before data can be exchanged. As discussed previously, initial ATM networks will have Permanent Virtual Channels (PVCs). Later networks will allow connections to be created when needed through a Signaling protocol based on the emerging CCITT Q.93B standard.  A further tutorial will examine Signaling in detail. Here are the capabilities that Phase 1 Signaling, defined by the ATM Forum UNI specification, will provide.

1. Demand (switched) channel connections.

2. Point-to-point and point-to-multipoint switched channel connections.

3. Connections with symmetric or asymmetric bandwidth requirements.

4. Single-connection point-to-point or point-to-multipoint calls.

5. Basic signaling functions via protocol messages, information elements and procedures.

6. VPCI/VPI/VCI assignment

7. A single, statically defined out-of-band channel for all signaling messages.

8. Error recovery.

9. Public UNI and Private UNI addressing formats for unique identification of ATM endpoints.

10. A client registration mechanism for exchange of addressing information across a UNI.

11. End-to-end Compatibility Parameter Identification.

The ATM Adaptation Layers

What is an Adaptation Layer?

| Described in previous articles, the PHY and ATM layers provide sequential transfer of fixed-size data units across a network. The ATM Adaptation Layer (AAL) adapts that transfer process to perform the upper layer services required by different users. The chart above shows four AAL types, each of which provides a different type of transport service. AAL1, 3/4, and 5 are described below. AAL2 is still in the standardization process. ATM Adaptation Type | 1 | 2 | 3/4 | 5 |
| --- | --- | --- | --- | --- |
| Service Examples | CBR Speech CBR Video DS3/1 Emulation | Variable Bit-Rate Video | SMDS Connectionless Service | TCP/IP Bursty LAN data |
| Timing Relationship? | Yes | Yes | No | No |
| Bit Rate | Constant | Variable | Variable | Variable |
| Multiple Simultaneous Packets on a Single VCC? | N/A | No | Yes | No |

AAL1

The ATM Adaptation Layer 1 performs the functions necessary to adapt constant bit rate (CBR) services to the ATM Layer services.  CBR voice, video and circuit emulation have certain characteristics in common:

- blocks of data appear at known, periodic intervals

- very intolerant of mis-sequenced data

- very intolerant of variation in delay

- somewhat tolerant of absolute delay

The basic AAL1 protocol shown to the right is very simple, with a single header preceding 47 bytes of data. The header's main job is to provide (error protected) sequence information for easy monitoring of cell loss or mis-sequencing. At the ATM layer, test equipment can monitor for cell delay variation, cell loss, and cell mis-sequencing to verify the quality of service across a network for AAL1.

AAL3/4

AAL3/4 provides the transport of variable length frames with error detection. Since ATM is a connection-oriented technology, AAL3/4 most naturally provides connection-oriented service, but can also be used to provide connectionless service with the aid of a Connectionless Server. This is where ATM and SMDS converge.  Users of AAL3/4 will typically have

- bursty LAN traffic

- data is reasonably tolerant of variation in delay

- router may have several simultaneous datagrams to send across a single VC

Here are some points of interest in the AAL3/4 figure. The "MID" is a Message Identifier, which allows multiple simultaneous packets on a single VC. The "BOM", "COM", and "EOM" are beginning/continuation/end of message indicators for each packet. Connectionless service via an AAL3/4 must be provided by routing packets entering the network to a Connectionless Server, which could for instance provide SMDS. This server would then perform the routing of the packet through the connection-oriented ATM network. A single preassigned VPI/VCI address for all SMDS service assures that packets entering the network from any point will be routed to the nearest server.

AAL5

AAL5 is a simplification of AAL3/4 which limits the transport to connection oriented and single frame at a time. This removes the need for MID, COM, BOM, and EOM, which increases the efficiency of the protocol and makes its implementation much easier. There still is a need to recognize the frame boundaries, and this is done by using one of the Payload Type bits in the ATM header, indicated as "MORE" and "LAST" in the figure.